DAA Assignment 2

DESIGN AND ANALYSIS OF ALGORITHMS

ASSIGNMENT 2 – CO2

CO2 Coverage: Brute-Force and Exhaustive-Search techniques – Selection Sort, Bubble Sort, Insertion Sort, Sequential Search, Brute-Force String Matching, Closest Pair, Convex Hull and Exhaustive Search.

Nature of Assignment: Individual

Duration: 1 Week

Total Marks: 50

Submission Date: 30/10/2026

# Q1 – Sorting Under Different Data Conditions (6 Marks)

## 1.1 Objective

To compare Selection Sort, Bubble Sort and Insertion Sort when dispatch-priority scores arrive in different arrangements, and to select an appropriate algorithm for a fulfilment centre whose existing orders are approximately 95% sorted.

## 1.2 Algorithm overview and predictions

| Algorithm | Main operation | Expected behaviour |
| --- | --- | --- |
| Selection Sort | Find the minimum in the unsorted suffix and place it at the front. | Always performs n(n−1)/2 comparisons. Input order has little effect on comparisons; at most n−1 swaps. |
| Bubble Sort (with early stop) | Compare adjacent elements and swap out-of-order neighbours. | Nearly sorted or sorted data can finish quickly; reverse order requires many comparisons and swaps. |
| Insertion Sort | Insert each item into its correct position in the sorted prefix. | Very efficient for sorted/nearly sorted data; worst case is reverse order with Θ(n²) shifts. |

## 1.3 Test datasets

| Test | Input list | Purpose |
| --- | --- | --- |
| A – Already sorted | [10, 20, 30, 40, 50, 60, 70, 80] | Best case for Bubble and Insertion Sort. |
| B – Reverse sorted | [80, 70, 60, 50, 40, 30, 20, 10] | Worst-case arrangement for Bubble and Insertion Sort. |
| C – Nearly sorted | [10, 20, 30, 50, 40, 60, 70, 80] | Only one adjacent pair is out of order; models small batches added to an ordered list. |
| D – Edge case | [5, 5, 5, 5, 5, 5, 5, 5] | Checks duplicate values and stability-related behaviour. |

## 1.4 Operation-count interpretation

For n = 8, Selection Sort always makes 8×7/2 = 28 comparisons. Its number of swaps depends on the implementation and input; the standard version performs no more than 7 swaps. Insertion Sort makes 7 comparisons on already sorted data and 7 shifts are not required. On the nearly sorted list, it makes 8 key comparisons (including the final failed comparison for the displaced 50) and shifts one element. On reverse order it performs 28 key comparisons and 28 shifts. Bubble Sort with an early-exit flag makes 7 comparisons and 0 swaps for sorted data; reverse order requires 28 comparisons and 28 swaps. The nearly sorted list needs 14 comparisons over two passes and one swap.

| Input condition (n=8) | Selection comparisons | Bubble comparisons / swaps | Insertion comparisons / shifts |
| --- | --- | --- | --- |
| Sorted | 28 | 7 / 0 | 7 / 0 |
| Reverse sorted | 28 | 28 / 28 | 28 / 28 |
| Nearly sorted | 28 | 14 / 1 | 8 / 1 |
| All equal | 28 | 7 / 0 | 7 / 0 |

The counts above use Bubble Sort with an early-stop flag and the usual insertion-sort loop that stops when the preceding key is not greater than the current key. Exact comparison counts can differ slightly if an implementation counts loop-condition checks as well as key comparisons.

## 1.5 Recommendation

Insertion Sort is recommended for the stated operating condition. When approximately 95% of records are already in priority order and only small batches are appended, the number of shifts is expected to be small. Insertion Sort takes close to linear time when the number of inversions is small, whereas Selection Sort still scans the unsorted suffix on every pass. Bubble Sort with early termination can also perform well on almost-sorted input, but Insertion Sort generally moves each new item directly into its correct position and is a natural fit for incremental additions. For large, fully unsorted datasets, a more scalable O(n log n) sorting method would normally be considered.

## 1.6 Python implementation

def selection_sort(a):

a = a[:]

comparisons = swaps = 0

for i in range(len(a) - 1):

m = i

for j in range(i + 1, len(a)):

comparisons += 1

if a[j] < a[m]:

m = j

if m != i:

a[i], a[m] = a[m], a[i]

swaps += 1

return a, comparisons, swaps

def bubble_sort(a):

a = a[:]

comparisons = swaps = 0

for end in range(len(a) - 1, 0, -1):

changed = False

for j in range(end):

comparisons += 1

if a[j] > a[j + 1]:

a[j], a[j + 1] = a[j + 1], a[j]

swaps += 1

changed = True

if not changed:

break

return a, comparisons, swaps

def insertion_sort(a):

a = a[:]

comparisons = shifts = 0

for i in range(1, len(a)):

key = a[i]

j = i - 1

while j >= 0:

comparisons += 1

if a[j] > key:

a[j + 1] = a[j]

shifts += 1

j -= 1

else:

break

a[j + 1] = key

return a, comparisons, shifts

# Q2 – When Is Sequential Search an Appropriate Engineering Choice? (4 Marks)

## 2.1 Method

Sequential Search checks records from the first position until the requested Tool ID is found or the list ends. It requires no sorting or index construction, which is useful because the tool-issue list changes frequently.

## 2.2 Experiment design and comparison counts

| List size | Successful: first | Successful: middle | Successful: last | Unsuccessful |
| --- | --- | --- | --- | --- |
| 10 | 1 | 5 | 10 | 10 |
| 100 | 1 | 50 | 100 | 100 |
| 1,000 | 1 | 500 | 1,000 | 1,000 |

For a successful search at position k (counting the first record as position 1), the number of comparisons is k. An unsuccessful search examines all n records, so it requires n comparisons. The table uses the middle position as floor((n+1)/2).

## 2.3 Engineering decision

Reasonable choice: small temporary lists, infrequent lookups, records frequently inserted/deleted, or situations where maintaining a sorted structure/index would cost more than scanning.

Stop recommending it: large lists with frequent lookup requests, especially when most searches are unsuccessful or the target is commonly near the end. Average successful-search cost is approximately (n+1)/2 comparisons for uniformly distributed positions; unsuccessful searches cost n.

Changed condition – 20 facilities: the combined record volume and request frequency increase. A central indexed data store or hash-based lookup keyed by Tool ID is preferable. Sequential Search may remain suitable for each small local list or as a fallback, but not as the main system-wide lookup method.

## 2.4 Python implementation

def sequential_search(records, target):

comparisons = 0

for index, record in enumerate(records):

comparisons += 1

if record["tool_id"] == target:

return index, comparisons

return -1, comparisons

# Q3 – Engineer the Behaviour of Brute-Force String Matching (6 Marks)

## 3.1 Algorithm

The pattern is aligned at every possible starting position in the text. Characters are compared from left to right. On a mismatch, the pattern shifts one position to the right; on a complete match, the starting index is returned.

## 3.2 Designed test conditions

| Test | Text and pattern | Prediction / reason |
| --- | --- | --- |
| A – Early match | Text = 'ALERT: login failed'; Pattern = 'ALERT' | Only five character comparisons are needed because the pattern matches at index 0. |
| B – Repeated-prefix stress | Text = 'A' repeated 20 times; Pattern = 'AAAAAB' | No match. Each of 15 alignments compares five A characters and then mismatches at B: 15×6 = 90 comparisons. |
| C – Match at final alignment | Text = 'A' repeated 20 times followed by 'B'; Pattern = 'AAAAAB' | The first 15 alignments each make 6 comparisons, then the final alignment matches in 6: 16×6 = 96 comparisons. |

## 3.3 Analysis

The test cases demonstrate that input length alone does not determine the work. The arrangement of characters matters. Repeated prefixes cause the algorithm to re-check the same text characters after every one-position shift. A long pattern with a long repeated prefix can therefore create many comparisons even when no match exists. For text length n and pattern length m, the worst-case comparison count is O((n−m+1)m), commonly written O(nm).

## 3.4 Python implementation

def brute_force_match(text, pattern):

comparisons = 0

n, m = len(text), len(pattern)

if m == 0:

return 0, comparisons

for start in range(n - m + 1):

j = 0

while j < m:

comparisons += 1

if text[start + j] != pattern[j]:

break

j += 1

if j == m:

return start, comparisons

return -1, comparisons

# Q4 – Closest Pair and the Cost of Examining Every Possibility (6 Marks)

## 4.1 Brute-force design

For every point i, compare it with every later point j where j > i. This visits each unordered pair exactly once and avoids duplicate comparisons such as (A,B) and (B,A).

The number of unique pairs among n drones is:

C(n, 2) = n(n − 1) / 2

For each pair, calculate Euclidean distance: d = √((xᵢ−xⱼ)² + (yᵢ−yⱼ)²). Keep the pair with the smallest distance. Comparing squared distances avoids unnecessary square-root calculations; take one square root for the final answer.

| Number of drones (n) | Unique pairs | Distance calculations |
| --- | --- | --- |
| 10 | 45 | 45 |
| 100 | 4,950 | 4,950 |
| 500 | 124,750 | 124,750 |

## 4.2 Scalability

The number of calculations grows quadratically, Θ(n²). Doubling the number of drones produces approximately four times as many pair checks. For this report, an illustrative acceptable response-time target is 1 second on the test computer. The exact drone-count boundary must be established by timing the implementation on the actual machine; it cannot be stated reliably without measured hardware-specific results. The inefficiency comes from checking every possible pair, including many pairs that cannot plausibly be closest.

## 4.3 Python implementation

def closest_pair(points):

if len(points) < 2:

return None, None, 0

best_sq = float("inf")

best_pair = None

checks = 0

for i in range(len(points) - 1):

for j in range(i + 1, len(points)):

checks += 1

dx = points[i][0] - points[j][0]

dy = points[i][1] - points[j][1]

d_sq = dx * dx + dy * dy

if d_sq < best_sq:

best_sq = d_sq

best_pair = (points[i], points[j])

return best_pair, best_sq ** 0.5, checks

# Q5 – Derive a Convex Hull Algorithm from a Geometric Principle (7 Marks)

## 5.1 Geometric decision rule

For two candidate points A(x₁,y₁) and B(x₂,y₂), and another point P(x,y), compute the signed cross product:

cross(A,B,P) = (x₂−x₁)(y−y₁) − (y₂−y₁)(x−x₁)

A positive value means P lies on one side of the directed line AB; a negative value means it lies on the other side; zero means collinear. If all other points lie on one side (or on the line), AB is a supporting boundary line and can contribute to the convex hull.

## 5.2 Procedure and pseudocode

HULL_EDGES(points):

edges = empty set

for each pair of distinct points A, B:

positive = false

negative = false

for each point P other than A and B:

s = cross(A, B, P)

if s > 0: positive = true

if s < 0: negative = true

if not (positive and negative):

add the undirected edge {A, B} to edges

return edges

The accepted supporting edges are then ordered around the boundary (for example, by computing the centroid of the hull vertices and sorting their polar angles). Collinear points on a boundary line should be handled carefully: retain only the extreme endpoints for a simple polygon boundary, or explicitly document whether boundary-collinear stations are included.

## 5.3 Test configurations

| Test | Coordinates | Expected boundary |
| --- | --- | --- |
| A – Interior points | (0,0), (4,0), (4,4), (0,4), (2,2), (1,2) | Square with four corner vertices; the two interior points are excluded. |
| B – Collinear challenge | (0,0), (2,0), (4,0), (4,3), (0,3), (2,1) | Rectangle boundary. The middle point (2,0) is collinear on an edge and is not a polygon corner; (2,1) is interior. |

## 5.4 Complexity and correctness

There are Θ(n²) candidate point pairs, and each pair is tested against Θ(n) other points. Therefore, the direct supporting-line test takes O(n³) time. An accepted line has no input point strictly on both sides. It is therefore a supporting line of the point set; its segment between the extreme points on that line forms a boundary edge of the convex hull. Degenerate cases such as fewer than three distinct points and all-collinear input should be handled separately.

## 5.5 Python implementation (supporting edges)

def cross(a, b, p):

return ((b[0] - a[0]) * (p[1] - a[1])

- (b[1] - a[1]) * (p[0] - a[0]))

def brute_force_hull_edges(points):

pts = list(dict.fromkeys(points))

edges = set()

n = len(pts)

if n < 2:

return edges

for i in range(n):

for j in range(i + 1, n):

pos = neg = False

for k in range(n):

if k == i or k == j:

continue

value = cross(pts[i], pts[j], pts[k])

if value > 0: pos = True

elif value < 0: neg = True

if not (pos and neg):

edges.add((pts[i], pts[j]))

return edges

Note: The implementation returns supporting point-pairs. If three or more points are collinear on one hull side, it can return multiple overlapping subsegments. For a final visual polygon, identify the extreme endpoint pair on each supporting line, remove duplicate/collinear intermediate vertices, and order the remaining vertices around the centroid.

# Q6 – Exhaustive Search and Combinatorial Growth (8 Marks)

## 6.1 Representation and strategy

Represent a package combination as a binary decision vector. Bit i = 1 means package i is selected; bit i = 0 means it is not selected. Enumerate all 2ⁿ subsets, calculate total weight and value, reject subsets whose weight exceeds the helicopter payload, and retain the feasible subset with the greatest value.

## 6.2 Small correctness-verification instance

| Package | Weight | Humanitarian value |
| --- | --- | --- |
| P1 | 2 | 6 |
| P2 | 3 | 10 |
| P3 | 4 | 12 |
| P4 | 5 | 15 |

Payload limit = 7. The best feasible combination is P2 + P3: total weight 3 + 4 = 7 and total value 10 + 12 = 22. P1 + P4 has weight 7 and value 21; P1 + P2 has value 16; P3 + P4 exceeds the limit. Checking all 16 subsets confirms that P2 + P3 is optimal.

## 6.3 Candidate growth

| Packages n | Subsets examined (2ⁿ) | Increase from previous row |
| --- | --- | --- |
| 8 | 256 | — |
| 16 | 65,536 | 256 times |
| 20 | 1,048,576 | 16 times |

Each additional package doubles the number of possible subsets. The growth is exponential, Θ(2ⁿ). Even a modest increase in package count can multiply the work because every package introduces a new include/exclude decision.

## 6.4 Deployment boundary

Choose a response-time requirement of 1 second as an illustrative planning target. The largest safe n must be measured on the target computer using representative package data. The table shows the exact candidate counts, but runtime depends on implementation, hardware and pruning. Exhaustive search is appropriate for small n when an exact optimum is essential; for larger n, use a redesigned exact or approximate method.

## 6.5 Python implementation

def exhaustive_knapsack(packages, capacity):

# Each package is (package_id, weight, value)

n = len(packages)

best_value = -1

best_selection = []

subsets_checked = 0

for mask in range(1 << n):

subsets_checked += 1

total_weight = 0

total_value = 0

chosen = []

for i in range(n):

if mask & (1 << i):

pid, weight, value = packages[i]

total_weight += weight

total_value += value

chosen.append(pid)

if total_weight <= capacity and total_value > best_value:

best_value = total_value

best_selection = chosen

return best_selection, best_value, subsets_checked

# Q7 – Diagnose, Redesign and Validate (13 Marks)

## Selected solution: Brute-Force String Matching

This solution is selected because Q3 demonstrates repeated comparisons caused by overlapping pattern prefixes. The same text characters are compared again after each one-position shift.

## A. Diagnose the computational bottleneck (2 marks)

In the repeated-prefix test, text = A repeated 20 times and pattern = AAAAAB. There are 15 possible alignments. At every alignment, five A characters match and the sixth comparison fails (B versus A), producing 15 × 6 = 90 comparisons. The bottleneck is repeated rechecking of a prefix that has already been matched.

## B. Define the design objective (2 marks)

Reduce repeated character comparisons while preserving exact-match correctness. For the stress test, the measurable objective is to use fewer than 90 character comparisons and avoid restarting the pattern comparison from its first character after every mismatch.

## C. Revised strategy – reuse matched-prefix information (4 marks)

Build a Longest Proper Prefix which is also Suffix (LPS) table for the pattern. When a mismatch occurs after j characters have matched, use LPS[j−1] to determine how many leading pattern characters are already known to match. Do not move the text index backwards. This reuses previous comparison information. Preprocessing costs O(m) time and O(m) extra memory; the search then takes O(n) time.

KMP_SEARCH(text, pattern):

lps = BUILD_LPS(pattern)

i = 0                 # text index

j = 0                 # pattern index

while i < length(text):

if text[i] == pattern[j]:

i = i + 1

j = j + 1

if j == length(pattern):

return i - j

else if j != 0:

j = lps[j - 1]   # reuse known prefix

else:

i = i + 1

return -1

## D. Prediction before implementation (1 mark)

The KMP strategy is predicted to use substantially fewer comparisons on repeated-prefix inputs. The expected search complexity changes from O(nm) in the brute-force worst case to O(n) after O(m) preprocessing.

## E. Implementation and validation (2 marks)

def build_lps(pattern):

lps = [0] * len(pattern)

length = 0

i = 1

while i < len(pattern):

if pattern[i] == pattern[length]:

length += 1

lps[i] = length

i += 1

elif length != 0:

length = lps[length - 1]

else:

lps[i] = 0

i += 1

return lps

def kmp_search(text, pattern):

if pattern == "":

return 0

lps = build_lps(pattern)

i = j = comparisons = 0

while i < len(text):

comparisons += 1

if text[i] == pattern[j]:

i += 1

j += 1

if j == len(pattern):

return i - j, comparisons

elif j != 0:

j = lps[j - 1]

else:

i += 1

return -1, comparisons

Validation procedure: run both algorithms on the exact same stress-test text and pattern; record the returned index and character-comparison count. Both must return -1 for the no-match case. Also test a successful pattern and verify that both return the same starting index. KMP preprocessing comparisons should be recorded separately if the comparison metric includes all work.

## F. Defend the final design (2 marks)

Expected benefit: KMP avoids rechecking text positions and has linear worst-case search time.

New cost: it builds an LPS array and uses O(m) additional memory.

When the original may be preferable: very short patterns, tiny log strings, or one-off searches where preprocessing overhead is not worthwhile.

Conclusion: KMP directly addresses the diagnosed repeated-prefix bottleneck. The final claim should be supported by actual comparison counts from running both implementations on the same test cases.

# APPENDIX – Consolidated Experimental Checklist

Run each supplied Python function using the listed test inputs.

For Q1, verify that all three sorting methods return the same sorted list; record comparisons and swaps/shifts.

For Q2, run searches for first, middle, last and absent Tool IDs at each list size.

For Q3 and Q7, record the exact text, pattern, match index and character comparisons; include KMP preprocessing separately.

For Q4, verify that the number of examined pairs equals n(n−1)/2 for each test size.

For Q5, plot the input points and accepted hull boundary; include at least one interior-point and one collinear challenge case.

For Q6, independently verify the small four-package optimum and record the subset count for each n.

Measure execution time several times on the same laptop and report the median; do not use timing as the only evidence.

# OVERALL CONCLUSION

The experiments and mathematical analysis show that algorithm selection must consider input arrangement, data size, request frequency and the structure of the problem. Insertion Sort is suitable for nearly ordered data; Sequential Search is acceptable for small, infrequently searched lists; brute-force string matching, closest-pair search, convex-hull testing and exhaustive subset enumeration become expensive as their candidate spaces grow. The redesign in Q7 illustrates how identifying and reusing previously established information can reduce repeated work. Engineering recommendations should be supported by operation counts and measurements collected on the target system.